MODERN COMPUTING

PRACTICAL SUBJECT CODE:

PSIT202

PSIT20topology2- Modern Computing

| Sr. No. | Title of Practical | Pg. No. | Date | Sign |
| --- | --- | --- | --- | --- |
| 1 | Configure IP SLA tracking and path control topology. |  | 17/04/2023 |  |
| 2 | Implementation of BGP using AS_path attribute. |  | 13/03/2023 |  |
| 3 | Configuring IBGP and EBGP sessions. |  | 08/05/2023 |  |
| 4 | Secure Management Plane. |  | 14/03/2023 |  |
| 5 | Configure and verify path control using PBR (Policy Based Routing) |  | 18/04/2023 |  |
| 6 | Demonstrate inter vlan routing. |  | 11/04/2023 |  |
| 7 | Simulating MP LS environment. |  | 08/05/2023 |  |
| 8 | Simulating VRF (Virtual Routing and Forwarding). |  | 08/05/2023 |  |

CERTIFICATE

This is to certify that the practical in the subject of 
PSIT202 Modern Computing
have been completed satisfactorily by

Mr./Ms.________________________________________

Roll no. _________________________in

practical fulfilment of M.Sc. I.T. degree (Sem II) examination for Academic Year 2022 – 23.

______________________

Practical Instructor

Ms.

Date:   /   / 2022

| _______________________ | _______________________ | _______________________ |
| --- | --- | --- |
| Internal Examiner | External Examiner | Principal Dr Rohini Kelkar |
| Date: / / 2022 | Date: / / 2022 | Date: / / 2022 |

PRACTICAL 1

Aim: Configure IP SLA tracking and path control topology.

Topology:

Steps/Commands:

Step1: Configure routers and connect serial links.

R1# conf t

(config) int s1/0

(config-if) ip add 209.165.201.1 255.255.255.0

no sh

int s1/1

ip add 209.165.202.1 255.255.255.0

no sh

int lo0

ip add 192.168.1.1 255.255.255.0

do sh ip int br | include up

R2# conf t

(config) int s1/0

(config-if) ip add 209.165.201.2 255.255.255.0

no sh

int s1/2

ip add 209.165.200.2 255.255.255.0

no sh

do sh ip int br | include up

R3# conf t

(config) int s1/1

(config-if) ip add 209.165.202.3 255.255.255.0

no sh

int s1/2

ip add 209.165.200.3 255.255.255.0

no sh

do sh ip int br | include up

Step 2: Configure static routing in branch router and dynamic routing using EIGRP (Enhanced Interior Gateway Routing Protocol) on ISP1 & ISP2 router.

(To not sum common IP address headers, use no auto-summary)

R1> exit

(config) ip route 0.0.0.0 0.0.0.0 209.165.201.0

R2> exit

(config) router eigrp 1

network 209.165.200.0 0.0.0.255

network 209.165.201.0 0.0.0.255

no auto-summary

R3> exit

(config) router eigrp 1

network 209.165.202.0 0.0.0.255

network 209.165.200.0 0.0.0.255

no auto-summary

R2> exit

ip route 192.168.1.0 255.255.255.0 209.165.201.1

R3> exit

ip route 192.168.1.0 255.255.255.0 209.165.202.1

R1> exit

do ping 209.165.200.3

R3> (config)

do ping 209.165.201.1

R2> do ping 192.168.1.1

R3> do ping 192.168.1.1

Task3: Configure IP SLA probes at branch routers.

R3> hostname isp2

R2> hostname isp1

R1> hostname branch

branch> (config)

ip sla 11

icmp-echo 209.165.201.2

frequency 10

exit

ip sla schedule 11 life forever start-time now

do sh ip sla configuration 11

do sh ip sla statistics

ON ALL ROUTERS (to save config):

do wr

Output:

PRACTICAL 2

Aim: Implementation of BGP using AS_path attribute.

Topology:

Steps/Commands:

Step1: Configuration

R2# conf t

int s1/0

ip add 192.168.1.2 255.255.255.0

no sh

R1# conf t

int s1/0

ip add 192.168.1.1 255.255.255.0

no sh

int s1/1

ip add 172.24.1.1 255.255.255.0

no sh

R3# conf t

int s1/1

ip add 172.24.1.3 255.255.255.0

no sh

Step2: Loopback

R2# int lo0

ip add 10.1.1.1 255.255.255.0

R1# int lo0

ip add 10.2.2.2 255.255.255.0

R3# int lo0

ip add 10.3.3.3 255.255.255.0

Step3: Configure as-bgp

R2# router bgp 100

neighbor 192.168.1.1 remote-as 300

network 10.1.1.0 mask 255.255.255.0

R1# router bgp 300

neighbor 192.168.1.2 remote-as 100

neighbor 172.24.1.3 remote-as 200

network 10.2.2.0 mask 255.255.255.0

R3# router bgp 200

neighbor 172.24.1.1 remote-as 300

network 10.3.3.0 mask 255.255.255.0

ON ALL ROUTERS:

do sh ip route

Step4: ping routers

R1#do ping 10.3.3.3 source lo0

R3#do ping 10.2.2.2 source lo0

Output:

PRACTICAL 3

Aim: Configuring IBGP and EBGP sessions.

Topology:

Steps/Commands:

Step 1: Drag and drop R1, R2 and R3; take 3 Ethernet switch and perform configurations on given routers.

R1# conf t

int f0/1

ip add 192.168.1.1 255.255.255.0

no sh

int s1/0

ip add 172.16.1.1 255.255.255.0

no sh

int s1/1

ip add 172.16.5.1 255.255.255.0

no sh

R2# conf t

int f0/0

ip add 10.10.10.2 255.255.255.0

no sh

int f0/1

ip add 192.168.2.2 255.255.255.0

no sh

int s1/0

ip add 172.16.1.2 255.255.255.0

no sh

R3# conf t

int f0/0

ip add 10.10.10.3 255.255.255.0

no sh

int f0/1

ip add 192.168.3.3 255.255.255.0

no sh

int s1/1

ip add 172.16.5.3 255.255.255.0

no sh

ON ALL ROUTERS:

do sh ip int br | include up

Step 2: Configure IRP(Interior Routing Protocol [using OSPF]) in autonomous system 65200 (AS65200)

R2(config)

router ospf 1

network 10.10.10.0 0.0.0.255 area 0

network 192.168.2.0 0.0.0.255 area 1

R3#(config)

router ospf 1

network 10.10.10.0 0.0.0.255 area 0

network 192.168.3.0 0.0.0.255 area 2

ON BOTH ROUTERS:

R3>(config)

do ping 192.168.2.2

R2>(config)

do ping 192.168.3.3

Step 3: IBGP and EBGP configurations

R1>(config)

router bgp 65100

network 192.168.1.0

network 172.16.1.0 mask 255.255.255.0

network 172.16.5.0 mask 255.255.255.0

neighbor 172.16.1.2 remote-as 65200

neighbor 172.16.5.3 remote-as 65200

R2>(config)

router bgp 65200

network 172.16.1.0 mask 255.255.255.0

redistribute ospf 1

neighbor 172.16.1.1 remote-as 65100

neighbor 10.10.10.3 remote-as 65200

R3>(config)

router bgp 65200

network 172.16.5.0 mask 255.255.255.0

redistribute ospf 1

neighbor 172.16.5.1 remote-as 65100

neighbor 10.10.10.2 remote-as 65200

Step 4: Final output:

(ON ALL ROUTERS)

(config) do sh ip route

(DO THIS ONLY WHEN EXAMINER ASKS)

R1> (config)

do ping 192.168.2.2

do ping 192.168.3.3

Output:

PRACTICAL 4

Aim: Secure management plane.

Topology:

Steps/Commands:

Step 1: Configure routers.

R1> conf t

int s1/0

ip add 10.1.1.1 255.255.255.0

no sh

int lo1

ip add 192.168.1.1 255.255.255.0

R2> conf t

int s1/0

ip add 10.1.1.2 255.255.255.0

no sh

int s1/1

ip add 10.2.2.2 255.255.255.0

no sh

R3> conf t

int s1/1

ip add 10.2.2.3 255.255.255.0

no sh

int lo1

ip add 192.168.2.1 255.255.255.0

Step2: Configure Routing:

R1> ip route 0.0.0.0 0.0.0.0 10.1.1.2

R2> ip route 192.168.1.0 255.255.255.0 10.1.1.1

ip route 192.168.2.0 255.255.255.0 10.2.2.3

R3> ip route 0.0.0.0 0.0.0.0 10.2.2.2

R1> do ping 192.168.2.1

R3> do ping 192.168.1.1

(START ONLY WHEN 100% SUCCESS ON PING)

Step3: Secure management access

R1> (config)

hostname r1

security password min-length 10

enable secret class12345

line console 0

password ciscoconpass

exec-timeout 5 0

login

logging synchronous

exit

line vty 0 4

password ciscovtypass

exec-timeout 5 0

login

exit

line aux 0

no exec

end

do wr

conf t

service password-encryption

banner motd $Unauthorized access not allowed$

exit

R3> (config)

hostname r3

security password min-length 10

enable secret class12345

line console 0

password ciscoconpass

exec-timeout 5 0

login

logging synchronous

exit

line vty 0 4

password ciscovtypass

exec-timeout 5 0

login

exit

line aux 0

no exec

end

do wr

conf t

(config)

service password-encryption

banner motd $Unauthorized access not allowed$

exit

r2> telnet 10.1.1.1

Output:

PRACTICAL 5

Aim: Configure and verify path control using PBR

(Policy Based Routing).

Topology:

Steps/Commands:

Step1: Perform IP configuration.

R1> #conf t

hostname r1

int s1/0

ip add 172.16.12.1 255.255.255.0

bandwidth 128

no sh

int s1/1

ip add 172.16.13.1 255.255.255.0

bandwidth 64

no sh

int lo0

ip add 192.168.1.1 255.255.255.0

exit

do sh ip int br | include up

R2> #conf t

hostname r2

int s1/0

ip add 172.16.12.2 255.255.255.0

bandwidth 128

no sh

int s1/2

ip add 172.16.23.2 255.255.255.0

bandwidth 128

no sh

int lo0

ip add 192.168.2.2 255.255.255.0

exit

do sh ip int br | include up

R3> #conf t

hostname r3

int s1/1

ip add 172.16.13.3 255.255.255.0

bandwidth 64

no sh

int s1/2

ip add 172.16.23.3 255.255.255.0

bandwidth 128

no sh

int s1/3

ip add 172.16.34.3 255.255.255.0

bandwidth 64

no sh

int lo0

ip add 192.168.3.3 255.255.255.0

do sh ip int br | include up

R4> #conf t

int s1/3

ip add 172.16.34.4 255.255.255.0

bandwidth 64

no sh

int lo0

ip add 192.168.4.1 255.255.255.0

int lo1

ip add 192.168.5.1 255.255.255.0

exit

do sh ip int br | include up

Step 2: Configure EIGRP on all routers.

R1> (config) router eigrp 1

network 172.16.12.0 0.0.0.255

network 172.16.13.0 0.0.0.255

network 192.168.1.0

no auto-summary

R2> (config) router eigrp 1

network 172.16.12.0 0.0.0.255

network 172.16.23.0 0.0.0.255

network 192.168.2.0

no auto-summary

R3> (config) router eigrp 1

network 172.16.13.0 0.0.0.255

network 172.16.23.0 0.0.0.255

network 172.16.34.0 0.0.0.255

network 192.168.3.0

no auto-summary

R4> (config) router eigrp 1

network 172.16.34.0 0.0.0.255

network 192.168.4.0

network 192.168.5.0

no auto-summary

Step 3: check the network:

'do sh ip route' on all routers

R1> do ping 192.168.4.1

R4> do ping 192.168.1.1

USE TRACE ROUTE COMMAND TO VERIFY PATH from R4 to R1 using loopback.

R4> do traceroute 192.168.1.1 source 192.168.4.1

R4> do traceroute 192.168.1.1 source 192.168.5.1

Step 4: Configure PBR to providepath control all traffic from source 192.168.5.1 should take the path r4 -> R3 -> R1, whereas traffic from 192.168.4.1 should take the path R4 -> R3 -> R2 -> R1

Step4: Perform PBR on RECEIVING ROUTER

R3# (config) ip access-list standard pbr-acl

permit 192.168.5.0 0.0.0.255

exit

route-map r3-to-r1 permit

match ip address pbr-acl

set ip next-hop 172.16.13.1

exit

int s1/3

ip policy route-map r3-to-r1

end

R4> do traceroute 192.168.1.1 source 192.168.4.1

R4> do traceroute 192.168.1.1 source 192.168.5.1

Output:

PRACTICAL 6

Aim: Demonstrate inter vlan routing.

Topology:

Steps/Commands:

(CISCO packet tracer)

Goto options -> preferences and check for link lights and port labels

Step1: check VLAN config in each switch

type command for all switches:

en

show vlan br

CHECK IF ALL SWITCHES HAVE SAME VLAN (1002,1003,1004,1005...)

Step2: disable all ports on all the switches.

commands for all switches:

conf t

interface range fa0/1-24

shutdown

interface range gi0/1-2

shutdown

Step3: Perform basic switch configurations like assign name to switches, password to switches as well as gateways.

hostnames: s0, s1 and s2

commands for all switches:

exit

(config)

hostname s0

enable secret class

no ip domain-lookup

ip default-gateway 172.17.99.1

line console 0

password cisco

login

line vty 0 15

password cisco

login

end

Step4: On the interfaces of the switch 2 connect it to the PCs, configure access mode and enable them.

commands for s2:

(config)

int fa0/11

(config-if)

switchport mode access

no shutdown

int fa0/12

switchport mode access

no shutdown

int fa0/13

switchport mode access

no shutdown

Step5: Configure IP addresses on the three PCs and the server.

PC0-> Desktop -> IP config

IP: 172.17.10.21 255.255.255.0

Default gateway: 172.17.10.1

PC1-> Desktop -> IP config

IP: 172.17.20.22 255.255.255.0

Default gateway: 172.17.20.1

PC2-> Desktop -> IP config

IP: 172.17.30.23 255.255.255.0

Default gateway: 172.17.30.1

Server -> Desktop -> IP config

IP: 172.17.50.254 255.255.255.0

Default gateway: 172.17.50.1

Step6: Configure VTP protocol on the switches.

s0 will be VTP server, s1 & s2 will be VTP client

s0:

Password: cisco

en Password: class

en

(#)

Password:

conf t

(config)

vtp mode server

vtp domain vsit

vtp password cisco

s1:

Password:

en

#

Password:

conf t

(config)

vtp mode client

vtp domain vsit

vtp password cisco

s2:

Password:

en

#

Password:

conf t

(config)

vtp mode client

vtp domain vsit

vtp password cisco

Step7: Configure trunking codes on all connections between switches and enable them.

s0:

(config)

int range fa0/1-3

(config-if)

switchport mode trunk

switchport trunk native vlan 99

no shutdown

int range fa0/5-6

switchport mode trunk

switchport trunk native vlan 99

no shutdown

s2:

(config)

int range fa0/3-6

(config-if)

switchport mode trunk

switchport trunk native vlan 99

no sh

s1:

(config)

int range fa0/1-4

(config-if)

switchport mode trunk

switchport trunk native vlan 99

no sh

s0:

(config-if-range)

exit

(config)

hostname management

(config-vlan)

vlan 10

name staff

vlan 20

name students

vlan 30

name guests

exit

do sh vlan br (On s0 and s2)

Step8: Configure interface vlan 99 on all the switches.

s0:

(config)

int vlan 99

(config-if)

ip add 172.17.99.11 255.255.255.0

end

s2:

(config)

int vlan 99

(config-if)

ip add 172.17.99.12 255.255.255.0

end

s1:

(config)

int vlan 99

(config-if)

ip add 172.17.99.13 255.255.255.0

end

Step9: Configure vlan 10, vlan 20 and vlan 30 on switch 2.

s2:

(config)

int fa0/11

(config-if)

switchport access vlan 10

int fa0/12

switchport access vlan 20

int fa0/13

switchport access vlan 30

Step10: perform configuration on router.

First set on R1 f0/0 with 172.17.50.1 and subnet mask and click ‘on’.

Router:

en

conf t

hostname r1

no ip domain-lookup

line console 0

(config-line)

password cisco

login

line vty 0 15

password cisco

login

end

conf t

(config)

enable secret class

int fa0/1

no sh

int fa0/1.1

(config-subif)

encapsulation dot1q 1

ip add 172.17.1.1 255.255.255.0

int fa0/1.10

encapsulation dot1q 10

ip add 172.17.10.1 255.255.255.0

int fa0/1.20

encapsulation dot1q 20

ip add 172.17.20.1 255.255.255.0

int fa0/1.30

encapsulation dot1q 30

ip add 172.17.30.1 255.255.255.0

int fa0/1.99

encapsulation dot1q 99 native

ip add 172.17.99.1 255.255.255.0

Step11: ping/deliver packets.

from PCs to Server

Output:

Practical 7

Aim: Simulating MP LS environment

Topology:

(before)

(after)

Steps/Commands:

Step 1: Configure routers

R1> conf t

int lo0

ip add 1.1.1.1 255.255.255.255

ip ospf 1 area 0

int f0/0

ip add 10.0.0.1 255.255.255.0

no sh

ip ospf 1 area 0

R2> conf t

int lo0

ip add 2.2.2.2 255.255.255.255

ip ospf 1 area 0

int f0/0

ip add 10.0.0.2 255.255.255.0

no sh

exit

ip ospf 1 area 0

int f0/1

ip add 10.0.1.2 255.255.255.0

no sh

ip ospf 1 area 0

R3> conf t

int lo0

ip add 3.3.3.3 255.255.255.255

ip ospf 1 area 0

int f0/1

ip add 10.0.1.3 255.255.255.0

no sh

ip ospf 1 area 0

Step 2: Verify connections.

R1>(config) do sh ip ospf int br

do sh ip int br include up

do ping 3.3.3.3 source lo0

R3>(config) do sh ip ospf int br

do sh ip int br | include up

do ping 1.1.1.1 source lo0

Step 3: Configure MPLS

(On ALL routers R1, R2 and R3)

R1,R2,R3> router ospf 1

mpls ldp autoconfig

R2> do sh mpls interface

do sh mpls ldp neigh

Step 4: Configuring VPN

R1> do traceroute 3.3.3.3

router bgp 1

neighbor 3.3.3.3 remote-as 1

neighbor 3.3.3.3 update-source lo0

no auto-summary

address-family vpnv4

neighbor 3.3.3.3 activate

R3> do traceroute 1.1.1.1

router bgp 1

neighbor 1.1.1.1 remote-as 1

neighbor 1.1.1.1 update-source lo0

no auto-summary

address-family vpnv4

neighbor 1.1.1.1 activate

(on R1 and R3)

R1,R3> do sh bgp vpnv4 unicast all summary

(On R1 and R2)

R1,R2,> do sh ip route

Outputs:

Practical 8

Aim: Simulating VRF (Virtual Routing and Forwarding).

Topology:

(before)

(after)

Steps/Commands:

(Perform MPLS)

Step 1: Adding Routers and Configuring VRF (Virtual Routing and Forwarding)

DRAG & DROP router R4, R5

R1>

int f0/1

ip add 192.168.1.1 255.255.255.0

no sh

exit

ip vrf RED

rd 4:4

route-target both 4:4

int f0/1

ip vrf forwarding RED

int f0/1

ip add 192.168.1.1 255.255.255.0

ip ospf 2 area 2

do sh ip ospf int br

do sh ip int br | include up

R3>

int f0/0

ip add 172.168.1.3 255.255.255.0

no sh

exit

ip vrf BLUE

rd 5:5

route-target both 5:5

int f0/0

ip vrf forwarding BLUE

int f0/0

ip add 172.168.1.3 255.255.255.0

ip ospf 3 area 3

do sh ip ospf int br

do sh ip int br | include up

R4> conf t

int lo0

ip add 4.4.4.4 255.255.255.255

ip ospf 2 area 2

int f0/1

ip add 192.168.1.4 255.255.255.0

no sh

ip ospf 2 area 2

int lo0

ip ospf 2 area 2

do sh ip ospf int br

do sh ip int br | include up

R5> conf t

int lo0

ip add 5.5.5.5 255.255.255.255

ip ospf 3 area 3

int f0/0

ip add 172.168.1.5 255.255.255.0

no sh

ip ospf 3 area 3

int lo0

ip ospf 3 area 3

do sh ip ospf int br

do sh ip int br | include up

Step 2: Verfiy connection

R1>(config) do sh ip route vrf RED

R3>(config) do sh ip route vrf BLUE

Outputs: